module i2c_addr_translator(

input  wire clk,          // FPGA system clock

input  wire rst_n,        // Asynchronous active-low reset

// Upstream I2C (master side)

input  wire sda_in,

input  wire scl_in,

output reg  sda_out,

// Downstream I2C (device side)

output reg  sda_to_dev,

output reg  scl_to_dev,

input  wire sda_from_dev

);

// -------------------------------------------------------------------------

// Parameters

// -------------------------------------------------------------------------

parameter [6:0] VISIBLE_ADDR = 7'h54;  // Address visible to main I2C master

parameter [6:0] ACTUAL_ADDR  = 7'h60;  // Real address of device

// -------------------------------------------------------------------------

// Internal Registers and Signals

// -------------------------------------------------------------------------

reg [7:0] shift_reg;

reg [2:0] bit_counter;

reg start_flag, stop_flag;

// FSM state encoding

reg [2:0] curr_state, next_state;

localparam IDLE        = 3'd0,

ADDR        = 3'd1,

CHECK       = 3'd2,

PASS_WRITE  = 3'd3,

PASS_READ   = 3'd4,

WAIT_STOP   = 3'd5;

// -------------------------------------------------------------------------

// START and STOP condition detection

// -------------------------------------------------------------------------

always @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

start_flag <= 1'b0;

stop_flag  <= 1'b0;

end else begin

// Simplified detection logic

if (scl_in && (sda_in == 1'b0))

start_flag <= 1'b1;

else

start_flag <= 1'b0;

if (scl_in && (sda_in == 1'b1))

stop_flag <= 1'b1;

else

stop_flag <= 1'b0;

end

end

// -------------------------------------------------------------------------

// FSM state transition

// -------------------------------------------------------------------------

always @(posedge clk or negedge rst_n) begin

if (!rst_n)

curr_state <= IDLE;

else

curr_state <= next_state;

end

always @(*) begin

next_state = curr_state;

case (curr_state)

IDLE:

if (start_flag)

next_state = ADDR;

ADDR:

if (bit_counter == 3'd7)

next_state = CHECK;

CHECK:

if (shift_reg[7:1] == VISIBLE_ADDR)

next_state = PASS_WRITE;

else

next_state = WAIT_STOP;

PASS_WRITE:

if (stop_flag)

next_state = IDLE;

PASS_READ:

if (stop_flag)

next_state = IDLE;

WAIT_STOP:

if (stop_flag)

next_state = IDLE;

default:

next_state = IDLE;

endcase

end

// -------------------------------------------------------------------------

// Address Shift and Replacement

// -------------------------------------------------------------------------

always @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

shift_reg   <= 8'h00;

bit_counter <= 3'd0;

end else begin

case (curr_state)

ADDR: begin

shift_reg   <= {shift_reg[6:0], sda_in};

bit_counter <= bit_counter + 1'b1;

end

CHECK: begin

if (shift_reg[7:1] == VISIBLE_ADDR)

shift_reg[7:1] <= ACTUAL_ADDR;

end

default: begin

bit_counter <= 3'd0;

end

endcase

end

end

// -------------------------------------------------------------------------

// Forward I2C Lines (simplified simulation version)

// -------------------------------------------------------------------------

always @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

sda_out    <= 1'b1;

sda_to_dev <= 1'b1;

scl_to_dev <= 1'b1;

end else begin

scl_to_dev <= scl_in; // Forward clock directly

case (curr_state)

PASS_WRITE: begin

sda_to_dev <= sda_in;

sda_out    <= 1'b1; // release bus

end

PASS_READ: begin

sda_out    <= sda_from_dev;

sda_to_dev <= 1'b1;

end

default: begin

sda_to_dev <= 1'b1;

sda_out    <= 1'b1;

end

endcase

end

end

endmodule