# FPGA-Based I²C Address Translator

**Author:** Kavya Sree

**Date:** November 2025

**Platform:** Xilinx Vivado / EDA Playground

**Company Task:** Vicharak LLP FPGA Internship Assignment

---

## 🎯 Objective

To design an **FPGA-based I²C Address Translator** that allows a device with a fixed I²C address to coexist with others that share the same default address.

The translator acts as an **I²C slave** to the main master and as an **I²C master** to the target device.

It dynamically remaps the visible address to a different one, while keeping all data intact.

---

## ⚙️ Design Overview

### **Top-Level Behavior**

- The module monitors the I²C bus and detects START and STOP conditions.

- When it recognizes the master accessing the *visible address*, it replaces that address with the *actual address* of the device.

- All read/write data transactions are transparently forwarded between the master and device.

---

## 🧩 Architecture

### **Main Blocks**

1. **Start/Stop Detector**

- Detects when an I²C transaction begins or ends based on SDA and SCL transitions.

2. **Shift Register + Bit Counter**

- Captures the address bits serially as they arrive.

- After receiving all 7 address bits, the FSM compares them to the “visible” address.

3. **Finite State Machine (FSM)**

- Controls how the translator reacts to different bus phases.

- FSM States:

- `IDLE` – Waiting for start condition

- `ADDR` – Capturing address bits

- `CHECK` – Compare and translate

- `PASS_WRITE` – Forward master → device data

- `PASS_READ` – Forward device → master data

- `WAIT_STOP` – Wait for stop condition

4. **Forwarding Logic**

- Controls the SDA/SCL direction depending on whether the master is writing or reading.

- Always forwards clock (`scl_in → scl_to_dev`).

---

## 🧠 FSM Flow (Simplified)